Public contract entitled:

Thickness Monitor for MBE chamber                   (OP JAK - FerrMion)

The subject of the procurement procedure is the supply of a Thickness Monitor for MBE system* (Molecular Beam Epitaxy).

A Thickness Monitor with two separate front sensor heads is required to measure the deposition rates and thicknesses of deposited materials (co-deposited fullerenes and metals for synthesis of ferroics). The Thickness Monitor will be used in the MBE chamber, the sensors will need to be parallel to the substrate (parallel to the ‘cooling water pipes’), and UHV compatible. In order to bring the sensors as close as possible to the substrate, it is necessary that the length of the sensors in vacuum (in the MBE chamber) is at least 20 cm.

The contracting authority sets the following minimum technical conditions:

| Subject | Parameter / Required technical and functional properties, value | The specific parameters of the offered solution or the supplier meets the conditions YES / NO | Offered solution - manufacturer, exact type leading to identification (or web link) |
| --- | --- | --- | --- |
| Monitor Thickness system (1 pc) | UHV compatible |  |  |
|  | min. 2 measuring channels |  |  |
|  | outputs for deposition rates and thicknesses of deposited materials |  |  |
|  | co-deposition monitoring capabilities |  |  |
|  | min. 2 front load single sensors |  |  |
|  | standard sensor heads parallel to the water line |  |  |
|  | DN40CF mounting flange with 2 water line cooling |  |  |
|  | in vacuum length min. 20 cm |  |  |
|  | Warranty period min. 12 months |  |  |
|  | Transport to the Contracting authority headquarter |  |  |

| Total offer price including transport (in EUR without VAT) | ……..EUR |
| --- | --- |
| Estimated value of the subject of performance including transport to the headquarter (in CZK without VAT) | 120 000 CZK |

Note: the supplier must fill in all fields highlighted in yellow

* The MBE system will be used for the preparation of hybrid nanostructures based on fullerenes and ferroic nanoparticles. The MBE system will consist of a large-volume vacuum chamber with a connected system of active elements, a sample manipulator, a thickness monitor and a system for linear sample transfer.

_________________________________________

signature of an authorized person on behalf of the supplier