Appendix 1 -

Technical Specifications

A small-scale contract

Thermoluminescent detectors

Standard form

In Table No. 1 below, the economic operator will confirm whether the goods being offered comply with the requirements specified in the table by checking the appropriate box in the “Compliance” column; i.e. “Yes” if the offered item complies with the respective requirement, or “No” if the item does not comply.

In the columns "Offer price" and "Total offer price", the supplier shall also enter the price per piece and the price per item in total. The supplier shall also state the price of transport of the goods.

In conclusion, each economic operator shall complete the declaration stating that all goods will be supplied strictly in accordance with the proposed technical specifications.

Table No. 1 – Proposed technical specifications

| Item | Requirements | Compliance | Quantity | Offer price per piece in CZK without VAT | Offer price total in CZK without VAT |
| --- | --- | --- | --- | --- | --- |
| Thermoluminescent detector MTS-N | Chemical composition: LiF: Mg, Ti (natural abundance of lithium) Effective atomic number Z: 8.2 Detection threshold [mGy]: 10 Linearity range [Gy]: 5×10-5–5 Thermal fading [% at room temperature]: <5%/yr Form: solid disc Size: 4.5 mm diameter, 0.9 mm thickness Reusability: unlimited | YES ☐ / NO ☐ | 100 | …… | …… |
| Thermoluminescent detector MTS-6 | Chemical composition: LiF:Mg,Ti (6Li-enriched) Effective atomic number Z: 8.2 Detection threshold [mGy]: 10 Linearity range [Gy]: 5×10-5–5 Thermal fading [% at room temperature]: <5%/yr Form: solid disc Size: 4.5 mm diameter, 0.9 mm thickness Reusability: unlimited | YES ☐ / NO ☐ | 100 | …… | …… |
| Thermoluminescent detector MTS-7 | Chemical composition: LiF: Mg, Ti (7Li-enriched) Effective atomic number Z: 8.2 Detection threshold [mGy]: 10 Linearity range [Gy]: 5×10-5–5 Thermal fading [% at room temperature]: <5%/yr Form: solid disc Size: 4.5 mm diameter, 0.9 mm thickness Reusability: unlimited | YES ☐ / NO ☐ | 100 | …… | …… |
| Thermoluminescent detector MCP-N | Chemical composition: LiF: Mg, Cu, P (natural abundance of lithium) Effective atomic number Z: 8.2 Detection threshold [nGy]: 50 Linearity range [Gy]: 10-7–10 Thermal fading [% at room temperature]: <5%/yr Form: solid disc Size: 4.5 mm diameter, 0.9 mm thickness Reusability: unlimited | YES ☐ / NO ☐ | 100 | …… | …… |
| Thermoluminescent detector MCP-6 | Chemical composition: LiF: Mg, Cu, P (6Li-enriched) Effective atomic number Z: 8.2 Detection threshold [nGy]: 50 Linearity range [Gy]: 10-7–10 Thermal fading [% at room temperature]: <5%/yr Form: solid disc Size: 4.5 mm diameter, 0.9 mm thickness Reusability: unlimited | YES ☐ / NO ☐ | 100 | …… | …… |
| Thermoluminescent detector MCP-7 | Chemical composition: LiF: Mg, Cu, P (7Li-enriched) Effective atomic number Z: 8.2 Detection threshold [nGy]: 50 Linearity range [Gy]: 10-7–10 Thermal fading [% at room temperature]: <5%/yr Form: solid disc Size: 4.5 mm diameter, 0.9 mm thickness Reusability: unlimited | YES ☐ / NO ☐ | 100 | …… | …… |
| Transport costs |  |  | 1 | …… |  |
| Offer price total in CZK without VAT including transportation |  |  |  | …… |  |

I (we) the undersigned …… hereby declare that the economic operator ……, if selected by the Contracting Authority to perform the public contract in question, will deliver the goods strictly in accordance with the proposed technical specifications and technical parameters within the time frame of …… days after the conclusion of the Purchase Contract.

……………………………………………………..

Stamp and signature of the economic operator's representative