Annex – Technical Specification

Sample Form

The Contracting authority determines special technical terms for the subject of the public procurement order for the suppliers. The supplier shall meet the special technical terms specified in Annex no. 1 – Technical Specification of the Purchase Contract.

The supplier shall state in Table no. 1 below whether the goods offered by them meets the requirements specified in the columns of the table by ticking off the option “Yes” in the column “Compliance” when the offered goods comply with the requirement and the option “No” when the goods do not comply with the requirement.

If the supplier even once ticks off the “No” option in the table of technical terms they will be excluded from the procedure because of the non-compliance with the terms. If the supplier tick off the “Yes” option and it is identified in the course of the assessment of the bids that the offered goods do not comply with the requirements, the supplier may be excluded because of the non-compliance with and the violation of the assignment terms. If the bidder ticks off neither the “Yes” option nor the “No option they may be excluded because of the non-compliance with the assignment terms.

At the end of this form the supplier shall fill in the solemn declaration that the goods will be supplied by them precisely according to the technical terms and the periods for delivery.

| Requirements | Technical term | Compliance |
| --- | --- | --- |
| Compatibility with the collimator | Compatibility with the radial collimator described in Annex no. 1 – Technical terms | YES ☐ / NO ☐ |
| Oscillation axis | The oscillation axis of the oscillation system shall be balanced with the focus of the collimator | YES ☐ / NO ☐ |
| Oscillation system | The oscillation system shall secure the blurring of the projection of collimation apertures in the detected signal. | YES ☐ / NO ☐ |
| Process of oscillation | Oscillation shall take place at least across 3 apertures of the collimator | YES ☐ / NO ☐ |
| Distance between the arm and horizontal axis of the detector | The distance between the arm carrying the detector system and the central horizontal axis of the detector is 225 mm | YES ☐ / NO ☐ |
| Compatibility of the electrical part of the oscillation system | The oscillation system shall include a stepper motor with the encoder compatible with the regulator of stepper motors Nanotec SMCI 33-2 or SMCI 47-S-2 | YES ☐ / NO ☐ |
| Connection to the radial collimator and commissioning of the oscillation system |  | YES ☐ / NO ☐ |
| Transport of the equipment |  | YES ☐ / NO ☐ |

I (we) the undersigned (í) ……solemnly declare that the supplier ……in the event that they are selected by the Contracting authority for the said public procurement order,  will supply the goods precisely according to the required technical terms and offered technical parameters within the period of …… day after the signing of the Purchase contract.

……………………………………………………..

Stamp and signature of the authorised person from the supplier