# Technická specifikace měřicí techniky a jejich komponent

## Použití předmětu zakázky

## Měřicí technika a její komponenty budou použity pro sestavování měřící a diagnostické části vyvíjených laserově - ultrazvukových metod pro bezkontaktní a nedestruktivní diagnostiku a charakterizaci materiálu a hodnocení mechanických vlastností zkoumaných materiálů. Nové laserově-ultrazvukové měřící systémy umožní in-situ sledování strukturních procesů, probíhajících v materiálech při extrémních teplotách a při mechanickém zatěžování.  Dodané jednotlivé měřící komponenty by měly zaručovat rychlé a spolehlivé ovládání zapojených měřících přístrojů.

Dodavatel je povinen splnit technické podmínky uvedené v Příloze ke kupní smlouvě – Technická specifikace předmětu veřejné zakázky.

Dodavatel předloží naceněný položkový rozpočet této přílohy pod názvem " Technická specifikace předmětu plnění - položkový rozpočet", v němž nacení bez výjimky a bez výhrad všechny položky určené zadavatelem k nacenění. Cena zboží (bez DPH) musí zahrnovat dodání do místa plnění.

Seznam přístrojů a komponent (počet kusů):

- Embedded Real -Time Controller with Reconfigurable FPGA – 1 kus

- Multifunction I/O module – 1 kus

- Universal Analog Input Module – 1 kus

- High-Speed CAN Module – 1 kus

- Relay Output Module – 3 kusy

- Voltage Input module – 1 kus

- DeviceNet Interface – 1 kus

- Serial interface RS485/RS422 – 1 kus

- Serial interface RS232¨- 1 kus

- USB Multifunction I/O device – 8 kusů

Na závěr v tomto formuláři dodavatel vyplní čestné prohlášení o tom, že dodá zboží přesně dle požadovaných technických podmínek a termínu dodání.

## Požadavky na technické provedení - požadované parametry, popisy

1. Embedded Real -Time Controller with Reconfigurable FPGA

| Název parametru | Požadavky | Splňuje Ano ☐ / Ne ☐ |
| --- | --- | --- |
| a) Processor | Type: Xilinx Zynq-7000, XC7Z020 All Programmable SoC Architecture: ARM Cotex-A9 Speed: 667 MHz Cores 2 Flash reboot endurance: 100,000 cycles | Ano ☐ / Ne ☐ |
| b) Operating System | Supported operating systém: Linux Real-Time (32-bit) | Ano ☐ / Ne ☐ |
| c) Software requirements | Application software: LabVIEW LabVIEW 2014 or later | Ano ☐ / Ne ☐ |
| d) Memory | Nonvolatile memory: 512 MB Volatile memory (DRAM): 256 MB | Ano ☐ / Ne ☐ |
| e) Network | Network interface: 10Base-T, 100Base-T, 1000Base-T Ethernet Compatibility: IEEE 802.3 Communication rates: 10 Mbps, 100 Mbps, 1,000 Mbps autonegotiated Maximum cabling distance: 100 m/segment | Ano ☐ / Ne ☐ |
| f) Internal Real-Time Clock | Accuracy 5 ppm | Ano ☐ / Ne ☐ |
| g) USB Ports | USB device port: Type: USB 2.0 Hi-Speed, with standard B connector Maximum data rate: 480 Mbps | Ano ☐ / Ne ☐ |
|  | USB host port Type: USB 2.0 Hi-Speed, with standard A connector Maximum data rate: 480 Mbps | Ano ☐ / Ne ☐ |
| h) Reconfigurable FPGA | Type: Xilinx Zynq-7000, XC7Z020 All Programmable SoC Number of logic cells: 85,000 Number of flip-flops: 106,400 Number of 6-input LUTs: 53,200 Number of DSP slices (18 × 25 multipliers): 220 Available block RAM: 4480 kbits Number of DMA channels: 16 Number of logical interrupts: 32 | Ano ☐ / Ne ☐ |
| i) Power Requirements | Voltage input range: 9 VDC to 30 VDC Reverse-voltage protection: 30 VDC maximum Maximum power input, with four C Series modules: 25 W Maximum power input, without C Series modules: 17 W | Ano ☐ / Ne ☐ |
| j) Physical Characteristics | Weight (unloaded): 1048 g Dimensions (unloaded): 272.8 mm × 88.1 mm × 62.3 mm Screw-terminal wiring: - Gauge: 0.2 mm 2 to 2.1 mm2 (24 AWG to 14 AWG) copper conductor wire - Wire strip length: 6 mm of insulation stripped from the end - Temperature rating: 85 °C - Torque for screw terminals: 0.20 Nm to 0.25 Nm - Wires per screw terminal: One wire per screw terminal Connector securement: - Securement type: Screw flanges provided - Torque for screw flanges: 0.3 Nm to 0.4 Nm | Ano ☐ / Ne ☐ |

2. Multifunction I/O module

| Název parametru | Požadavky | Splňuje |
| --- | --- | --- |
| a) Analog Input | Number of channels: 8 single-ended channels ADC resolution: 12 bits Type of ADC: Successive approximation register (SAR) Input range: 0 V to 5 V ±1% DNL: ±1.25 LSB Conversion time: 50 µs (20 kS/s) Input coupling: Input impedance: 1 MΩ in parallel with 50 pF Bandwidth: 1 kHz Stability: - Gain drift: 80 ppm/°C - Offset drift: 85 µV/°C | Ano ☐ / Ne ☐ |
| b) Analog Output | Number of channels: 8 channels DAC resolution: 12 bits Type of DAC: String Startup voltage: 0 V Output range: 0 V to 5 V ±1% Current drive: ±1 mA Output impedance: 5 Ω Update time: 50 µs (20 kS/s) Short-circuit protection: Indefinitely Slew rate: 30 V/ms Settling time: 900 µs DNL: ±1 LSB Capacitive drive: 1,500 pF Stability: - Gain drift: 85 ppm/°C - Offset drift: 180 µV/°C | Ano ☐ / Ne ☐ |
| c) Digital Input/Output | Number of channels: 4 channels Default power-on line direction: Input Input/output type: LVTTL, single-ended Digital logic levels: - Maximum input voltage: 5.2 V - Input high: 2 V - Input low: 0.8 V - Output high - Sourcing 100 µA: 2.7 V - Output low - Sinking 100 µA: 0.2 V Maximum I/O switching frequency: 1 MHz Capacitive drive: 100 pF | Ano ☐ / Ne ☐ |
| d) Power Requirements | Power consumption from chassis: - Active mode: 600 mW maximum - Sleep mode: 1 mW maximum Thermal dissipation (at 70 °C): - Active mode: 600 mW maximum - Sleep mode: 1 mW maximum | Ano ☐ / Ne ☐ |

3. Universal Analog Input Module

| Název parametru | Požadavky | Splňuje |
| --- | --- | --- |
| a) Circuitry | Voltage Circuitry Current Circuitry Thermocouple Circuitry 4-Wire Resistance and 4-Wire RTD Circuitry 3-Wire RTD Circuitry Full-Bridge Circuitry Half-Bridge Circuitry Digital In Circuitry Open Contact Circuitry | Ano ☐ / Ne ☐ |
| b) Physical Characteristics | Spring terminal wiring: - Gauge: 0.14 mm2 to 1.5 mm2 copper conductor wire - Wire strip length: 10 mm of insulation stripped from the end Temperature rating: 90 °C, minimum Wires per spring terminal: - One wire per screw terminal; two wires per screw terminal using a 2-wire ferrule Ferrules: - Single ferrule, uninsulated: - 0.14 mm2 to 1.5 mm2 (26 AWG to 16 AWG) 10 mm barrel length - Single ferrule, insulated: - 0.14 mm2 to 1.0 mm2 (26 AWG to 18 AWG) 12 mm barrel length - Two-wire ferrule, insulated - 2x 0.34 mm2 (2x 22 AWG) 12 mm barrel length Connector securement: - Securement type: Screw flanges provided - Torque for screw flanges: 0.2 Nm Weight 160 g | Ano ☐ / Ne ☐ |

c) Vstupní rozsahy

| Typ měření | Nominální rozsahy | Aktuální rozsahy | Splňuje |
| --- | --- | --- | --- |
| Voltage | ±60 V, ±15 V, ±4 V, ±1 V, ±125 mV | ±60 V, ±15 V, ±4 V, ±1 V, ±125 mV | Ano ☐ / Ne ☐ |
| Current | ±25 mA | ±25 mA | Ano ☐ / Ne ☐ |
| Thermocouple | ±125 mV | ±125 mV | Ano ☐ / Ne ☐ |
| 4- Wire and 2 – Wire Resistance | 10 kΩ, 1 kΩ | 10.5 kΩ, 1.05 kΩ | Ano ☐ / Ne ☐ |
| 4-Wire and 3-Wire RTD | Pt 1000, Pt 100 | 5.05 kΩ, 505 Ω | Ano ☐ / Ne ☐ |
| Quarter-Bridge | 350 Ω, 120 Ω | 390 Ω, 150 Ω | Ano ☐ / Ne ☐ |
| Half-Bridge | ±500 mV/V | ±500 mV/V | Ano ☐ / Ne ☐ |
| Full-Bridge | ±62.5 mV/V, ±7.8 mV/V | ±62.5 mV/V, ±7.8125 mV/V | Ano ☐ / Ne ☐ |
| Digital In | - | 0 V to 60 V | Ano ☐ / Ne ☐ |
| Open Contact | - | 1.05 kΩ | Ano ☐ / Ne ☐ |

d) Přesnost

| Typ měření | Rozsah | Odchylka měření | Posuv měření | Splňuje |
| --- | --- | --- | --- | --- |
|  |  | Hodnoty pro typickou teplotu (25°C ±5°C) Maximální teplotní provozní rozsah (-40°C to 70°C) |  |  |
| Voltage | ±60 V | ±0.3, ±0.4 | ±20, ±50 | Ano ☐ / Ne ☐ |
|  | ±15 V | ±0.3, ±0.4 | ±60, ±180 | Ano ☐ / Ne ☐ |
|  | ±4 V | ±0.3, ±0.4 | ±240, ±720 | Ano ☐ / Ne ☐ |
|  | ±1 V | ±0.1, ±0.18 | ±15, ±45 | Ano ☐ / Ne ☐ |
| Voltage Thermocouple | ±125 V | ±0.1, ±0.18 | ±120, ±360 | Ano ☐ / Ne ☐ |
| Current | ±25 mA | ±0.1, ±0.6 | ±30, ±100 | Ano ☐ / Ne ☐ |
| 4- Wire and 2 – Wire Resistance | 10 kΩ, | ±0.1, ±0.5 | ±120, ±320 | Ano ☐ / Ne ☐ |
|  | 1 kΩ | ±0.1, ±0.5 | ±1200, ±3200 | Ano ☐ / Ne ☐ |
| 4-Wire and 3-Wire RTD | Pt 1000 | ±0.1, ±0.5 | ±240, ±640 | Ano ☐ / Ne ☐ |
|  | Pt 100 | ±0.1, ±0.5 | ±2400, ±6400 | Ano ☐ / Ne ☐ |
| Quarter-Bridge | 350 Ω | ±0.1, ±0.5 | ±2400, ±6400 | Ano ☐ / Ne ☐ |
|  | 120 Ω | ±0.1, ±0.5 | ±2400, ±6400 | Ano ☐ / Ne ☐ |
| Half-Bridge | ±500 mV/V | ±0.03, ±0.07 | ±300, ±450 | Ano ☐ / Ne ☐ |
| Full-Bridge | ±62.5 mV/V | ±0.03, ±0.08 | ±300, ±1000 | Ano ☐ / Ne ☐ |
|  | ±7.8 mV/V | ±0.03, ±0.08 | ±2200, ±8000 | Ano ☐ / Ne ☐ |

e) Stabilita

| Typ měření | Rozsah | Rychlost zesílení | Posuv zesílení | Splňuje |
| --- | --- | --- | --- | --- |
| Voltage | ±60 V | ±20 | ±0.2 | Ano ☐ / Ne ☐ |
|  | ±15 V | ±20 | ±0.8 | Ano ☐ / Ne ☐ |
|  | ±4 V | ±20 | ±3.2 | Ano ☐ / Ne ☐ |
|  | ±1 V | ±10 | ±0.2 | Ano ☐ / Ne ☐ |
| Voltage Thermocouple | ±125 V | ±10 | ±1.6 | Ano ☐ / Ne ☐ |
| Current | ±25 mA | ±15 | ±0.4 | Ano ☐ / Ne ☐ |
| 4- Wire and 2 – Wire Resistance | 10 kΩ, | ±15 | ±3 | Ano ☐ / Ne ☐ |
|  | 1 kΩ | ±15 | ±30 | Ano ☐ / Ne ☐ |
| 4-Wire and 3-Wire RTD | Pt 1000 | ±15 | ±6 | Ano ☐ / Ne ☐ |
|  | Pt 100 | ±15 | ±60 | Ano ☐ / Ne ☐ |
| Quarter-Bridge | 350 Ω | ±15 | ±120 | Ano ☐ / Ne ☐ |
|  | 120 Ω | ±15 | ±240 | Ano ☐ / Ne ☐ |
| Half-Bridge | ±500 mV/V | ±3 | ±20 | Ano ☐ / Ne ☐ |
| Full-Bridge | ±62.5 mV/V | ±3 | ±20 | Ano ☐ / Ne ☐ |
|  | ±7.8 mV/V | ±3 | ±20 | Ano ☐ / Ne ☐ |

f) Šum

| Typ měření | Rozsah | Časování |  |  |  | Splňuje |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Vysokorychlostní | 60 Hz | 50 Hz | Vysoká přesnost |  |
| Voltage | ±60 V | 7.6 | 1.3 | 1.3 | 0.5 | Ano ☐ / Ne ☐ |
|  | ±15 V | 10.8 | 1.9 | 1.9 | 0.7 | Ano ☐ / Ne ☐ |
|  | ±4 V | 10.8 | 2.7 | 2.7 | 1.3 | Ano ☐ / Ne ☐ |
|  | ±1 V | 7.6 | 1.3 | 1.3 | 0.5 | Ano ☐ / Ne ☐ |
| Voltage Thermocouple | ±125 V | 10.8 | 1.9 | 1.9 | 1.0 | Ano ☐ / Ne ☐ |
| Current | ±25 mA | 10.8 | 1.9 | 1.9 | 1.0 | Ano ☐ / Ne ☐ |
| 4- Wire and 2 – Wire Resistance | 10 kΩ, | 4.1 | 1.3 | 0.8 | 0.3 | Ano ☐ / Ne ☐ |
|  | 1 kΩ | 7.1 | 1.8 | 1.2 | 0.7 | Ano ☐ / Ne ☐ |
| 4-Wire and 3-Wire RTD | Pt 1000 | 7.6 | 1.7 | 1.1 | 0.4 | Ano ☐ / Ne ☐ |
|  | Pt 100 | 10.8 | 1.9 | 1.9 | 0.9 | Ano ☐ / Ne ☐ |
| Quarter-Bridge | 350 Ω | 5.4 | 1.0 | 1.0 | 0.7 | Ano ☐ / Ne ☐ |
|  | 120 Ω | 5.4 | 1.0 | 1.0 | 0.7 | Ano ☐ / Ne ☐ |
| Half-Bridge | ±500 mV/V | 3.8 | 0.5 | 0.5 | 0.2 | Ano ☐ / Ne ☐ |
| Full-Bridge | ±62.5 mV/V | 5.4 | 1.0 | 1.0 | 0.8 | Ano ☐ / Ne ☐ |
|  | ±7.8 mV/V | 30 | 4.7 | 4.7 | 2.3 | Ano ☐ / Ne ☐ |

g) Polomůstkový měnič

| Typ měření | Základní odpor (Ω) | Vstupní napětí | Splňuje |
| --- | --- | --- | --- |
| Half-Bridge | 700 | 2.5 V | Ano ☐ / Ne ☐ |
|  | 240 | 2.0 V | Ano ☐ / Ne ☐ |

h) Plnomůstkový měnič, tříčtvrtečnímůstkový měnič, odporový měnič a termočlánkové měření

| Typ měření | Základní odpor (Ω) | Vstupní napětí | Splňuje |
| --- | --- | --- | --- |
| Full-Bridge | 350 | 2.7 V | Ano ☐ / Ne ☐ |
|  | 120 | 2.2 V | Ano ☐ / Ne ☐ |
| Quarter-Bridge, Resistance and RTD | 120 | 50 mV | Ano ☐ / Ne ☐ |
|  | 350 | 150 mV | Ano ☐ / Ne ☐ |
|  | 1.000 | 430 mV | Ano ☐ / Ne ☐ |
|  | 10.000 | 2200 mV | Ano ☐ / Ne ☐ |

4. High-Speed CAN Module

| Požadavky | Splňuje |
| --- | --- |
| 2-port, high-speed CAN module; Port 1 internally powered, Port 2 externally powered | Ano ☐ / Ne ☐ |
| Standard DE9M (DB9) male connector for each port | Ano ☐ / Ne ☐ |
| Hardware synchronization with any CompactRIO I/O module | Ano ☐ / Ne ☐ |
| ISO 11898 compliance for standard (11-bit) and extended (29-bit) arbitration IDs | Ano ☐ / Ne ☐ |
| Ability to transmit/receive 100 percent bus load at 1 Mbit/s | Ano ☐ / Ne ☐ |

5. Relay Output Modul

| Název parametru | Požadavky | Splňuje |
| --- | --- | --- |
| a) Output Characteristics | Number of channels: 4 electromechanical relay channels Relay type: Single pole single throw (SPST) Power-on output state: Channels off Switching capacity (resistive load): - Switching voltage: 60 VDC max, 250 Vrms max - Switching current, per channel - One channel on: 2.5 A max at 30 VDC, 1 A max at 60 VDC, 2.5 A max at 250 Vrms - Two channels on:2 A max at 30 VDC, 1 A max at 60 VDC, 2 A max at 250 Vrms - All channels on: 1.5 A max at 30 VDC, 1 A max at 60 VDC, 1.5 A max at 250 Vrms Resistance per channel, channel on: 0.2 Ω 14 Switching rate : 1 operation per second Relay release time: 10 ms max Relay operate time: 15 ms max Relay bounce time: 3 ms Off state leakage: 10 μA max Lice expectancy: - Mechanical (no load): 20,000,000 operations - Electrical (connecting to load) : 100,000 operations | Ano ☐ / Ne ☐ |

6. Voltage Input module

| Název parametru | Požadavky | Splňuje |
| --- | --- | --- |
| a) Input Characteristics | Number of Channels: 4 analog input channels ADC resolution: 16 bits Type of ADC: Successive approximation register (SAR) Input voltage ranges: - Minimum: ±10.5 V - Typical: ±10.6 V - Maximum: ±10.7 V Overvoltage protection: ±30 V Maximum Sampling Rate: - DAQ :1 MS/s 1 - FPGA User-Controlled I/O: 1 MS/s - FPGA I/O Nodes: 350 kS/s Stability: - Gain drift: 6 ppm/°C - Offset drift: 29 μV/°C CMRR (fin = 60 Hz): 100 dB -3 dB bandwidth: >1 MHz Input impedance: >1 GΩ Noise: 0.75 LSBrms Total Harmonic Distortion (THD) (20 Vpp at 1 kHz): -85 dB Crosstalk (20 Vpp at 1 kHz): -100 dB | Ano ☐ / Ne ☐ |

7. DeviceNet Interface

| Název parametru | Požadavky | Splňuje |
| --- | --- | --- |
| High-Speed CAN Characteristics | Transceiver: NXP PCA82C251T Max baud rate: 1 Mbps CAN_H, CAN_L bus lines voltage: -27 to +40 VDC CAN Supply voltage range (VSUP) +9 to +30 VDC MTBF Contact NI for Bellcore MTBF or MIL-HDBK-217F specifications. | Ano ☐ / Ne ☐ |

8. Seriál interface RS485/RS422

| Vlastnosti | RS - 422 | RS - 485 | Splňuje |
| --- | --- | --- | --- |
| Type of transmission lines | Differential | Differential | Ano ☐ / Ne ☐ |
| Maximum number of drivers | 1 | 32 | Ano ☐ / Ne ☐ |
| Maximum number of receivers | 10 | 32 | Ano ☐ / Ne ☐ |
| Maximum cable Length | 4.000 ft | 4.000 ft | Ano ☐ / Ne ☐ |
| Maximum CMV | ±7 V | +12 to -7 V | Ano ☐ / Ne ☐ |
| Driver output | 2 to 6 V | 1.5 to 6 V | Ano ☐ / Ne ☐ |
| Driver load | 100 Ω | 60 Ω | Ano ☐ / Ne ☐ |

| Název parametru | Požadavky | Splňuje |
| --- | --- | --- |
| Vlastnosti sběrnice | Temperature range: -40 to 70 °C Maximum baud rate: 3.6864 Mbps Maximum cable length: 1.2 km Data line ESD protection (human body model): ±15 kV MTBF: 514016 hours at 25 °C | Ano ☐ / Ne ☐ |

9. Seriál interface RS232

| Vlastnosti | RS-232 | Splňuje |
| --- | --- | --- |
| Type of transmission lines | Single ended | Ano ☐ / Ne ☐ |
| Maximum number of drivers | 1 | Ano ☐ / Ne ☐ |
| Maximum number of receivers | 1 | Ano ☐ / Ne ☐ |
| Maximum cable Length | 2.5 nF equivalent | Ano ☐ / Ne ☐ |
| Maximum CMV | ±25 V | Ano ☐ / Ne ☐ |
| Driver output | 5 to 25 V | Ano ☐ / Ne ☐ |
| Driver load | <3 kΩ | Ano ☐ / Ne ☐ |
| Temperature range | -40 to 70 °C | Ano ☐ / Ne ☐ |
| Maximum baud rate | 921.6 Mbps | Ano ☐ / Ne ☐ |
| Maximum cable length | 250pF equivalent | Ano ☐ / Ne ☐ |
| Data line ESD protection (human body model) | ±15 kV | Ano ☐ / Ne ☐ |
| MTBF | 448008 hours at 25 °C | Ano ☐ / Ne ☐ |

10. USB Multifunction I/O device

| Název parametru | Požadavky | Splňuje |
| --- | --- | --- |
| a) Analog Input | Number of channels: - Differential: 4 - Single-ende: 8 ADC resolution: 16 bit Maximum sample rate (aggregate): 50 kS/s Converter type: Successive approximation AI FIFO: 2,047 samples Trigger sources: Software, PFI 0, PFI 1 Input range: ±10 V Working voltage: ±10 V Overvoltage protection: - Powered-on: ±30 V - Powered-off: ±20 V Input impedance: >1 GΩ Input bias current: ±200 pA Absolute accuracy: - Typical at full scale: 6 mV - Maximum over temperature, full scale: 26 mV - System noise: 0.4 mVrms DNL:16-bit, no missing codes INL: ±1.8 LSB CMRR: 56 dB (DC to 5 kHz) Bandwidth: 300 kHz | Ano ☐ / Ne ☐ |
| b) Analog Output | Analog outputs: 2 DAC resolution: 16-bit Output range: ±10 V Maximum update rate: 5 kS/s simultaneous per channel, hardwaretimed AO FIFO: 2,047 samples Trigger sources: Software, PFI 0, PFI 1 Output current drive: ±5 mA Short circuit current: ±11 mA Slew rate: 3 V/μs Output impedance: 0.2 Ω Absolute accuracy (no load): - Typical at full scale: 8.6 mV - Maximum over temperature, full scale: 32 mV DNL: 16-bit, no missing codes INL: ±4 LSB Power-on state: 0 V Startup glitch: -7 V for 10 µs | Ano ☐ / Ne ☐ |
| c) Digital Input/Output | 13 digital lines: - Port 0: 8 lines - Port 1: 4 lines - Port 2: 1 lines Function: - P0.<0..7>: Static digital input/output - P1.0: Static digital input/output - P1.1/PFI 1: Static digital input/output, counter source or digital trigger - P1.<2..3>: Static digital input/output - P2.0/PFI 0: Static digital input/output, counter source or digital trigger Direction control: Each channel individually programmable as input or output Output driver type: Each channel individually programmable as open collector or active drive Absolute maximum voltage range:0.3 V to 5. 5 V with respect to D GND Pull-down resistor: 47.5 kΩ to D GND Power-on state: Input | Ano ☐ / Ne ☐ |
| d) Timebase | Timebase frequency: 80 MHz Timebase accuracy: ±100 ppm Timing resolution: 12.5 ns |  |
| e) Digital Input | Input voltage range (powered on): 0 to 5 V Input voltage range (powered off): 0 to 3.3 V Input voltage protection: 20 V on two lines per port (maximum of five lines for all ports) for up to 24 hours Minimum Vh: 2.3 V Maximum Vl: 0.8 V Maximum input leakage current: - At 3.3 V: 0.8 mA - At 5 V: 4.5 mA | Ano ☐ / Ne ☐ |
| f) Digital Output (Active Drive) | Maximum Vol (4 mA): 0.7 V Maximum Vol (1 mA): 0.2 V Minimum Voh (4 mA): 2.1 V Minimum Voh (1 mA): 2.8 V Maximum Voh: 3.6 V Maximum output current per line: ±4 mA | Ano ☐ / Ne ☐ |
| g) Digital Output (Open Collector) | Maximum Vol (4 mA): 0.8 V Maximum Vol (1 mA): 0.2 V Maximum output (sinking) current per line: -4 mA Maximum pull-up voltage: 5 V Maximum leakage current: - At 3.3 V: 0.8 mA - At 5 V: 4.5 mA | Ano ☐ / Ne ☐ |
| h) Counter | Number of counters: 1 Resolution: 32-bit Counter measurements: Edge counting, rising or falling Counter direction: Count up Counter source: PFI 0 or PFI 1 Maximum input frequency: 5 MHz Minimum high pulse width: 100 ns Minimum low pulse width: 100 ns | Ano ☐ / Ne ☐ |
| i) Power Source | Power Source +5 V Output voltage: +5 V, ±3% Maximum current: 150 mA Overcurrent protection: 200 mA Short circuit current: 50 mA Overvoltage protection: 20 V | Ano ☐ / Ne ☐ |
| j) Bus Interface | USB specification: USB Full Speed USB bus speed: 12 Mb/s | Ano ☐ / Ne ☐ |

Položkový rozpočet

| Pol. | Název | Počet | Cena Kč bez DPH |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1 ks | Celkem |
| 1 | Embedded Real -Time Controller with Reconfigurable FPGA | 1 ks | 0,00 Kč | 0,00 Kč |
| 2 | Multifunction I/O module | 1 ks | 0,00 Kč | 0,00 Kč |
| 3 | Universal Analog Input Module | 1 ks | 0,00 Kč | 0,00 Kč |
| 4 | High-Speed CAN Module | 1 ks | 0,00 Kč | 0,00 Kč |
| 5 | Relay Output Module | 3 ks | 0,00 Kč | 0,00 Kč |
| 6 | Voltage Input module | 1 ks | 0,00 Kč | 0,00 Kč |
| 7 | DeviceNet Interface | 1 ks | 0,00 Kč | 0,00 Kč |
| 8 | Serial interface RS485/RS422 | 1 ks | 0,00 Kč | 0,00 Kč |
| 9 | Serial interface RS232 | 1 ks | 0,00 Kč | 0,00 Kč |
| 10 | USB Multifunction I/O device | 8 ks | 0,00 Kč | 0,00 Kč |
| Doprava |  |  |  | 0,00 Kč |
| Celkem dodaného zboží bez DPH |  |  |  | 0,00 Kč |
| DPH |  |  |  | 0,00 Kč |
| Celkem s DPH |  |  |  | 0,00 Kč |

Já (my) níže podepsaný (í) [DOPLNÍ DODAVATEL] čestně prohlašuji (prohlašujeme), že dodavatel [DOPLNÍ DODAVATEL] v případě jeho výběru zadavatelem v předmětné veřejné zakázce dodá zboží přesně dle požadovaných technických podmínek a nabízených technických parametrů v termínu [DOPLNÍ DODAVATEL] dnů po podpisu Kupní smlouvy.

……………………………………………………..

Razítko a podpis oprávněné osoby dodavatele